Quotation:

HN-6025GA FIBER LASERMACHINE

| model | Standard Equipment |  |  | Power |  |  | Fiber Source | FOB Qingdao Price |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HN-6025GA | 1.6000*2500mm working surface exchange table with cover - Raycus fiber laser source CE with globe service - BOCI BLT642H laser head - Cypcut 8000C control system - Control cabinet independent - Air condition for cabinet of machine - Japan Yaskawa servo motor and driver Z: 750W, X: 2.9KW Y axis 4.4kw 8.22kwn16barn Other standard equipment ( such as: servo-motor,bearing etc.) |  |  | 12kw |  |  | Raycus | 85000$/set |  |
| Payment Terms; 30% in advance; 70% before delivery.. Payment; Price Validity; 20 days. Lead Time; 25 working days. Shipping; Shipping cost to Mersin Turkey; 00usd/container. Packing suggested; 40HQ*1. Shipping Time; 45 days arrive.. |
| Total amount CIF to Izmir Turkey : $ |  |  |  |  |  |  |  |  |  |
| Engineer Overseas Training Cost |  |  |  |  |  |  |  |  |  |
| 1. | Technician visa fee: $ RMB ($) |  |  |  |  |  |  |  |  |
| 2. | Training and maintain commission: $150/day/person( 5 to10 days) |  |  |  |  |  |  |  |  |
| 3. | International round-trip tickets: around $ |  |  |  |  |  |  |  |  |
| 4. | Abroad accommodation |  |  |  |  |  |  |  |  |
| Spare Parts for one Years’ Using |  |  |  |  |  |  |  |  |  |
| Items | Unite price |  | Suggested | lifetime |  |  |  | Amount |  |
| Nozzle | $15 |  | 10*pieces |  |  | 1 month |  |  | $150 |
| Protection lens | $20 |  | 5*pieces |  |  | 1 year |  |  | $100 |
| Focus lens | $180 |  | 1*piece |  |  | 1 year |  |  | $180 |
| Collimating lens | $180 |  | 1*piece |  |  | 1 year |  |  | $180 |
| Ceramic ring | $50 |  | 1*piece |  |  | 1 year |  |  | $50 |
| Remark: The quotation on page one include one set of spare parts and has been installed in laser head, besides, 9*nozzle and 4*protection lens are also included already in the quotation as replacement parts. |  |  |  |  |  |  |  |  |  |

- WARRANTY: machine  5years

- Leading time:in 25 working days

- METHOD OF PAYMENT:T/T

- TRADE TERMS:CIF

- PAYMENT REQUEST: Deposit 30% down on the contract price,70% payment before shipment

- QUOTATION VALIDITY: in 20 working days

- GUARANTEE:

(1)24 hours online service

(2)User-friendly English manual and Video CD for machine using and maintaining.

(3)overseas support

- machine details

The global exhibition

Ⅴ Machine Instruction

5.1 Introduction of HN-6025GA single table machine

Hongniu fiber laser cutting machine is a mature product that has reached the domestic advanced level. This series of products is recommended for the metal material processing industry. The bed is a honeycomb carbon structure, which has undergone aging annealing treatment. , The running speed of the machine is extremely fast, and the processing efficiency is greatly improved.

The cutting head has an automatic tracking system， adopts capacitive induction control, which makes cutting head kept same distance from workpiece surface, so that the distance between the nozzle and the surface of the workpiece that can finish not only flat metal sheet cutting but also some concave&convex  surface metal sheet cutting.

Fine cutting quality: non-contact cutting, edge cutting  gets less affected by heat, nearly no thermal deformation of the workpiece, slump formed can be avoided , almost linear cutting can be finished  by one time and no need repaet work。

Save mold investment: It can complete the processing of various complex structures by drawing any image on the computer for laser processing, no mold, no mold consumption, no need to repair the mold, save the time to replace the mold, which save the processing cost , reducing the production cost, especially suitable for the processing of large products.

- Material saving: Adopting computer programming,  which can nest different shapes of workpiece, improving material utilization.

5.2 Features

| - 5.2.1 Machine Frame - Adopt high-strength steel structure or aluminum alloy beam, light weight and good dynamic performance, rough machining after annealing to eliminate internal stress, and finishing after secondary vibration aging treatment to ensure the overall strength stability of the beam. |  |
| --- | --- |
|  | 5.2.2 Gantry Beam Adopting high-strength steel structure or aluminum alloy beam, lighter weight, better dynamic performance, rRough machining to relieve internal stress after annealing, finishing after secondary vibration aging treatment, which guarantee stability of whole gantry beam. |
| 5.2.3 Multi-function CNC Control System Cypcut control system, which has features such as intelligent diagnosis, multi-thread operation, three-level perforation, controlling laser energy , finding adaptive edge , corner acceleration and deceleration automatically, flying cutting, mechanical precision compensation, etc |  |
|  | 5.2.4 Servo motor and Driver High-performance servo motor adopts a one-step locking connector, which is highly resistant to vibration and shock. The debugging is convenient, operation is more stable, the positioning faster, which can match servo drive to control the position , speed and torque . |
| 5.2.5 Laser Source(Raycus Laser Source) Raycus fiber laser source supplier has R&D, producing, serving , to vertical integration capability from materials , parts, to whole laser source having global influence |  |
|  | 5.2.6 Laser Head 1. The cutting laser head has great advantages in medium power large platform cutting applications 2 laser head is completely sealed which avoiding optical parts polluted by dust 3. The laser head adopts two-point centering adjustment, focusing adopting imported motor drive, which improves the punching efficiency |
| HM22-16 Skid mounted screw air compressor |  |
|  | TODC-8B/11KW |

Technology Parameter

- 7.1 Configuration Parameter

| Item | Model | Name | Unit | Remark |
| --- | --- | --- | --- | --- |
| 1 | HN-6025GA double table with cover | Fiber laser cutting machine | 1set | Effective working area：6000mm*2500mm Cutting precision :（±0.02mm） |
| 2 | 12kw | Laser power source | 1 set | Brand: Raycus |
| 3 | Laser cutting head | High precision professional laser cutting head | 1 set | BOCI BLT642H |
| 4 | Control system | 8000C | 1 set | Cypcut control system |
| 5 | Y axis driver | Japan Yaskawa servo motor and driver 4.4kw | 2 sets | Brand: Japan Yaskawa |
| 6 | X axis driver | Japan Yaskawa servo motor and driver 2.9kw | 1 set | Brand: Japan Yaskawa |
| 7 | Z axis driver | Japan Yaskawa servo motor and driver 750w | 1 set | Brand: Japan Yaskawa |
| 8 | Reducer | Shimpo/ Motory | 3 sets | Japan Shimpo/France Motoreducer |
| 8 | Electrical components | original | 1 set | Schneier |
| 9 | Guide rail | Linear guide rail | 1 set | Brand Germany PEK |
| 10 | Racks | Grinding grade racks | 1 set | Brand Germany Lertesen |
| 11 | Proportional valve | Japan SMC | 1 set | original |
| 12 | Computer | Industrial grade | 1 set | Standard |
| 13 | Gantry beam | Integral Casting Aluminum | 1 set | Standard |
| 14 | Standard | Fiber laser chiller | 1 set | Hanli |
| 15 | Machine net weight | 14600kgs | Gross weight | 15000kgs |
| 16 | Stabilizer | 200KVA | 1 set |  |
| 17 | Air compressor | 37kw 16bar | 1 set | Optional(unit price is 6600 dollars) |
| 18 | Dust collector | TODC-8B/11KW | 1 set | Optional(unit price 5000 dollars) |
| Remark |  |  |  |  |

Sample：

Cutting Parameter12kw

| 激光器型号/fiber laser model |  | RFL-12000(100u) |  | 切割头光学配比/Cutting head optical ratio |  | 准直/Collimating lensF100/聚焦/focus lens F150 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 材料/material | 厚度/thickness [mm] | 速度/speed [m/min] | 功率/power [W] | 气体/assist gas | 气压/pressure [bar] | 喷嘴/nozzle type [mm] | 焦点位置/focus site [mm] | 切割高度/laser head height[mm] |
| 碳钢/carbon steel | 1 | 50-60 | 12000 | N₂/Air | 12 | 1.5S | 0 | 1 |
|  | 2 | 35-40 |  |  | 12 | 2.0S | 0 | 0.5 |
|  | 3 | 28-33 |  |  | 13 | 2.0S | 0 | 0.5 |
|  | 4 | 20-24 |  |  | 13 | 2.5S | 0 | 0.5 |
|  | 5 | 15-18 |  |  | 13 | 2.5S | 0 | 0.5 |
|  | 6 | 10-13 |  |  | 13 | 2.5S | 0 | 0.5 |
|  | 8 | 7-10 |  |  | 13 | 3.0S | -1.5 | 0.5 |
|  | 10 | 6-6.5 |  |  | 13 | 4.0S | -3 | 0.5 |
| 碳钢/carbon steel | 10 | 2-2.3 | 6000 | O₂正焦 | 0.6 | 1.2E | +6 | 0.8 |
|  | 12 | 1.8-2 | 7500 |  | 0.6 | 1.2E | +7 | 0.8 |
|  | 14 | 1.6-1.8 | 8500 |  | 0.6 | 1.4E | +7 | 0.8 |
|  | 16 | 1.5-1.6 | 9500 |  | 0.6 | 1.4E | +8 | 0.8 |
|  | 20 | 1.3-1.4 | 12000 |  | 0.6 | 1.6E | +8 | 0.8 |
|  | 22 | 0.9-1 |  |  | 0.7 | 1.8E | +9 | 0.8 |
|  | 22 | 1-1.2 |  |  | 0.7 | 1.4SP | +11 | 0.5 |
|  | 25 | 0.7-0.9 |  |  | 0.7 | 1.8E | +11 | 0.8 |
|  | 25 | 0.8-1 |  |  | 0.7 | 1.5SP | +12 | 0.5 |
|  | 30 | 0.4-0.5 |  |  | 1.3 | 1.8E | +11 | 1.2 |
|  | 30 | 0.7-0.8 |  |  | 0.8 | 1.5SP | +12 | 0.5 |
|  | 40 | 0.25-0.3 |  |  | 1.5 | 1.8E | +11.5 | 1.2 |
| 碳钢/carbon steel | 12 | 3-3.5 | 12000 | O₂负焦 | 1 | 1.6SP | -10 | 1.5 |
|  | 14 | 3-3.2 |  |  | 1 | 1.6SP | -10 | 1.5 |
|  | 16 | 2.8-3 |  |  | 1 | 1.6SP | -12 | 1.5 |
|  | 20 | 2-2.3 |  |  | 1.2 | 1.6SP | -12 | 1.5 |
|  | 25 | 1.1-1.3 |  |  | 1.3 | 1.8SP | -14 | 1.5 |
|  | 30 | 0.9-1 |  |  | 1.4 | 1.8SP | -14 | 1.5 |
| 不锈钢/stainless steel | 1 | 50-60 | 12000 | N₂ | 10 | 2.0S | 0 | 1 |
|  | 2 | 40-45 |  |  | 12 | 2.0S | 0 | 0.5 |
|  | 3 | 30-35 |  |  | 13 | 2.0S | 0 | 0.5 |
|  | 4 | 22-26 |  |  | 12 | 2.0S | 0 | 0.5 |
|  | 5 | 15-18 |  |  | 15 | 2.5S | 0 | 0.5 |
|  | 6 | 13-15 |  |  | 8 | 3.5B | 0 | 0.5 |
|  | 8 | 8-10 |  |  | 7 | 5.0B | 0 | 0.5 |
|  | 10 | 6.5-7.5 |  |  | 5 | 5.0B | -1 | 0.5 |
|  | 12 | 5-5.5 |  |  | 6 | 6.0B | -4 | 0.5 |
|  | 14 | 3-3.5 |  |  | 6 | 7.0B | -6 | 0.3 |
|  | 16 | 2-2.3 |  |  | 6 | 7.0B | -8 | 0.3 |
|  | 18 | 1.3-1.5 |  |  | 6 | 7.0B | -9 | 0.5 |
|  | 20 | 1.2-1.4 |  |  | 6 | 7.0B | -11 | 0.3 |
|  | 25 | 0.7-0.9 |  |  | 6 | 7.0B | -13 | 0.3 |
|  | 30 | 0.25-0.3 |  |  | 10 | 7.0B | +7 | 0.3 |
|  | 40 | 0.15-0.2 |  |  | 15 | 7.0B | +8 | 0.3 |
| 不锈钢/stainless steel | 1 | 50-60 | 12000 | Air | 10 | 2.0S | 0 | 1 |
|  | 2 | 40-45 |  |  | 10 | 2.5S | 0 | 0.5 |
|  | 3 | 30-35 |  |  | 10 | 2.5S | 0 | 0.5 |
|  | 4 | 22-28 |  |  | 10 | 3.5B | 0 | 0.5 |
|  | 5 | 16-19 |  |  | 10 | 3.5B | 0 | 0.5 |
|  | 6 | 14-17 |  |  | 10 | 3.5B | 0 | 0.5 |
|  | 8 | 9-11 |  |  | 10 | 3.5B | 0 | 0.5 |
|  | 10 | 7-8 |  |  | 10 | 3.5B | -1 | 0.5 |
|  | 12 | 5.5-6 |  |  | 10 | 5.0B | -4 | 0.5 |
|  | 14 | 3.5-4 |  |  | 10 | 5.0B | -6 | 0.5 |
|  | 16 | 2.2-2.4 |  |  | 10 | 5.0B | -8 | 0.5 |
|  | 18 | 1.3-1.6 |  |  | 10 | 5.0B | -9 | 0.5 |
|  | 20 | 1.2-1.5 |  |  | 10 | 5.0B | -11 | 0.3 |
|  | 25 | 0.7-1 |  |  | 10 | 5.0B | -13 | 0.3 |
|  | 30 | 0.3-0.6 |  |  | 10 | 5.0B | -14 | 0.3 |
| 铝合金 | 1 | 45-50 | 12000 | N₂ | 12 | 2.0S | 0 | 0.8 |
|  | 2 | 30-35 |  |  | 12 | 2.0S | -1 | 0.5 |
|  | 3 | 20-25 |  |  | 12 | 2.0S | -1 | 0.5 |
|  | 4 | 18-20 |  |  | 12 | 2.0S | -2 | 0.5 |
|  | 5 | 14-16 |  |  | 14 | 2.5S | -3 | 0.5 |
|  | 6 | 10-12 |  |  | 14 | 2.5S | -3 | 0.5 |
|  | 8 | 6-8 |  |  | 14 | 2.5S | -4 | 0.5 |
|  | 10 | 4-6 |  |  | 14 | 5.0B | -5 | 0.5 |
|  | 12 | 2-3 |  |  | 16 | 5.0B | -5 | 0.5 |
|  | 14 | 1.5-2.5 |  |  | 16 | 5.0B | -5 | 0.5 |
|  | 16 | 1.3-2 |  |  | 16 | 5.0B | -5 | 0.5 |
|  | 18 | 1-1.6 |  |  | 16 | 5.0B | -5 | 0.5 |
|  | 20 | 0.8-1.2 |  |  | 16 | 7.0B | -5 | 0.3 |
|  | 25 | 0.5-0.7 |  |  | 16 | 7.0B | -5 | 0.3 |
|  | 30 | 0.25-0.3 |  |  | 18 | 7.0B | +7 | 0.3 |
|  | 40 | 0.15-0.2 |  |  | 18 | 7.0B | +8 | 0.3 |
| 黄铜 | 1 | 35-45 | 12000 | N₂ | 12 | 2.0S | 0 | 1 |
|  | 2 | 30-35 |  |  | 12 | 2.0S | -1 | 0.5 |
|  | 3 | 18-22 |  |  | 12 | 2.0S | -1 | 0.5 |
|  | 4 | 15-18 |  |  | 12 | 2.0S | -2 | 0.5 |
|  | 5 | 12-15 |  |  | 14 | 2.5S | -3 | 0.5 |
|  | 6 | 8-10 |  |  | 14 | 2.5S | -3 | 0.5 |
|  | 8 | 5-7 |  |  | 14 | 2.5S | -4 | 0.5 |
|  | 10 | 4-5 |  |  | 14 | 5.0B | -5 | 0.5 |
|  | 12 | 1.8-2 |  |  | 14 | 5.0B | -5 | 0.5 |
|  | 14 | 1.2-1.4 |  |  | 16 | 5.0B | -8 | 0.5 |
|  | 16 | 0.8-1 |  |  | 16 | 5.0B | -11 | 0.3 |
| 紫铜 | 1 | 25-30 | 12000 | O₂ | 5 | 2.0S | -0.5 | 1 |
|  | 2 | 20-25 |  |  | 5 | 2.0S | -1 | 0.5 |
|  | 3 | 16-18 |  |  | 6 | 2.0S | -2 | 0.5 |
|  | 4 | 10-12 |  |  | 8 | 2.0S | -3 | 0.5 |
|  | 5 | 6-8 |  |  | 8 | 2.5S | -4.5 | 0.5 |
|  | 6 | 4-5 |  |  | 8 | 2.5S | -5 | 0.5 |
|  | 8 | 2-2.5 |  |  | 10 | 3.0S | -6 | 0.5 |
|  | 10 | 1-1.2 |  |  | 12 | 4.0S | -8 | 0.5 |
| 备注：表中红标参数为打样参数，在实际加工中受各类因素影响较大，仅适合小批量生产，不推荐大批量生产加工，建议使用更高功率激光器。 Note: The red marked parameters in the table are proofing parameters, which are greatly affected by various factors in actual processing. They are only suitable for small batch production. Mass production and processing are not recommended. It is recommended to use higher power lasers. |  |  |  |  |  |  |  |  |

Using cost 12kw

| 12000W 光纤机使用成本分析 12000W Fiber laser using cost |  |  |  |  |
| --- | --- | --- | --- | --- |
| 辅助气体 Assisted gas 能耗 Consumption |  | 可选方案一： 空气切割（用空压机组切割不锈钢板） ChooseⅠ:using air compressor Group as air supply to cutting Stainless steel | 可选方案二： 氧气切割（用 O2 切割不锈钢） ChooseⅡ:using O2 Cutting Stainless steel | 可选方案三： 氮气切割（用 N2 切割不锈钢） ChooseⅢ:using N2 Cutting Stainless steel |
| 电 力 消 耗Power Consumption | 激光器 Laser module | 36kw | 36kw | 36kw |
|  | 冷水机组 Water Chiller Group | 14kw | 14kw | 14kw |
|  | 机床主机 Host Machine | 16kw | 16kw | 16kw |
|  | 除尘设备 Dust Exhausting Equipment | 5kw | 5kw | 5kw |
| 易损件 Quick-wear Part |  | 2.5 RMB/h | 2.5 RMB/h | 2.5 RMB/h |
| 气体消耗 Gas Consumption |  | 15kw | 约（About） 25 RMB/h | 约（About）100 RMB/h |
| 总电力功率 Total Power |  | 86 kw | 71 kw | 71 kw |
| 平均功率（取 60％切割效率） Average Power Consunption (Take 60％Cutting Efficiency) |  | 86x60％=51.6 kw | 71x60％=42.6kw | 71x60％=42.6kw |
| 总运行成本 All Cost (按 1RMB/kwh) |  | 51.6+2.5=54.1 RMB/h | 42.6+2.5+25=70.1 RMB/h | 42.6+2.5+100=145.1 RMB/h |

Ⅷ、After Sale Service

Technology support

We will send our engineer to the customers’ place to help install,test the machine,and to train the customers’ technicians ,our engineer will stay at customers’ place for 5-7 days,customers should pay the round-trip air tickets/hotels/food fees/visa fee for our engineer. Training fee is               100 USD each day.

Exhibition support

If our agents need technical support at their show,our engineer will go to your show and work together,customers should pay the

round-trip air tickets/hotels/food fees/ visa fee/ technical support fee 100 USD each day.

24hours online service

The customers’ technicians can keep in touch with our engineer at any time,through wechat/whatsapp/mobile phone or email,our engineer will help to solve your problems as soon as possible.

THANK YOU FOR YOUR ATTENTION

JINAN HONGNIU MACHINERY EQUIPMENT CO.,LTD

Any question contact M.s Sophia Liu

Email: sales06@hongniulaser.com

Wechat:lasermachine2025